THE INFORMATION PROVIDED IS BELIEVED TO BE CORRECT BUT DOES NOT PURPORT TO BE ALL INCLUSIVE AND SHALL BE USED ONLY AS A GUIDE.  POLYMEDCO, INC. SHALL NOT BE HELD LIABLE FOR ANY DAMAGE RESULTING FROM HANDLING OR CONTACT WITH THE ABOVE PRODUCT.

R&D Director: _______________________________    Date: ____________________

# General Information

| Chemical Name & Synonyms: NOT APPPLICABLE | Trade Name & Catalog Number: ## Poly stat STREP A Test II Catalog Number: STR25 |
| --- | --- |
| Chemical Family: In Vitro Diagnostic Test Kits | Formula: NOT APPLICABLE |
| Proper DOT Shipping Name: NOT APPLICABLE | DOT Hazard Classification: NOT APPLICABLE |
| Contact: Polymedco, Inc 510 Furnace Dock Road Cortlandt Manor, NY 10567 | Contact Phone Number: (914) 739-5400 Contact Fax Number: (914)-739-0306 Poison Control Center: 1-800-876-4766 |

# Ingredients

| Item | Percent | Threshold Limit Value (units) |
| --- | --- | --- |
| Extraction Tablet- Sodium Nitrate Positive Control- Sodium Azide Heat-Inactivated Strep A Negative Control- Sodium Azide Heat-Inactivated Strep B Extraction Reagent Acetic Acid | >90% 0.1% N/A 0.1% N/A 0.5% | OSHA 15 mg/cubic meter (TWA) ACGIH 10mg/ml cubic meter (TWA) OSHA PEL 0.3 mg/cubic meter CL None established OSHA PEL 0.3 mg/cubic meter CL None established ACGIH 10ppm (25mg/cubic meter) |

# Physical Data

| Boiling Point (C): Sodium Azide: Not Available Acetic Acid: 117 C to 118 C Sodium Nitrate: Not Available | Specific Gravity (Water at 1.0 Sodium Azide: 1.850 Acetic Acid: 1.049 Sodium Nitrate: Not Available |
| --- | --- |
| Vapor Pressure (mm Hg): NOT APPLICABLE | Percent Volatile by Volume (%): NOT APPLICABLE |
| Vapor Density (Air at 1.0): NOT APPLICABLE | Evaporation Rate: NOT APPLICABLE |
| Appearance: Box with bottled reagents, strip, extraction tube, and swabs. | Odor: NONE KNOWN |

1 of 3

# Fire and Explosion Data

| Flash Point (Test Method): Not Applicable | Auto Ignition Temperature (F): Not Applicable |
| --- | --- |
| Flammable Limits: NOT APPLICABLE | LEL: NOT APPLICABLE UEL: NOT APPLICABLE |
| Extinguishing Media: Sodium Azide: Dry Chemical Powder Acetic Acid: Carbon Dioxide, Dry Chemical Powder or Appropriate Foam. Sodium Nitrate: Water Spray | Unusual Fire / Explosive Hazards: NONE KNOWN |
| Special Fire-Fighting Procedures: NON-FLAMMABLE |  |

# Health Hazard Data

| Threshold Limit Value: See Section II | OSHA Threshold Limit Value: See Section II |
| --- | --- |
| Carcinogen: NTP Program: Sodium Azide and Acetic Acid- Inconclusive | ACGIH Threshold Limit Value: See Section II |
| Carcinogen: IARC Program: Sodium Azide, Acetic Acid and Sodium Nitrate. | Symptoms of Exposure: Sodium Azide: Acute Effects: May be fatal if inhaled, swallowed , or absorbed through skin. May cause eye irritation. May cause skin irritation. Exposure can cause: Nausea, headache, and vomiting. Corrisive. Acetic Acid: Harmful if swallowed, inhaled, or absorbed through skin. Material is extremely destructive to tissue of the mucous membranes and upper respiratory tract, eyes and skin. Inhalation may be fatal as a result of spasm. Inflammation and edema symptoms of the larynx and bronchi. Chemical pneumonitis and pulmonary edema. Symptoms of exposure may include burning sensation coughing, wheezing, laryngitis, shortness of breath, headache, nausea and vomiting. Sodium Nitrate: Harmful if swallowed, inhaled, or absorbed through skin. Causes eye and skin irritation. Material is irritating to mucous membranes and upper respiratory tract. Absorption into the body leads to the formation of methemoglobin which in sufficient concentration causes cyansis. Onset may be delayed 2 to 4 hours or longer. |
| Medical Condition Aggravated by Exposure: Eyes/Skin Hypersensitivity | Primary Route(s) of Entry: Eye and Skin contact; Ingestion, Inhalation |
| 2 of 3 |  |
| FIRST AID: (Sodium Azide), (Acetic Acid) and (Sodium Nitrate): Immediately flush eyes or skin with copious amounts of water for at least 15 minutes while removing contaminated clothing and shoes. Assure adequate flushing of the eyes by seperating the eyelids with fingers. If inhaled: Remove to fresh air. If not breathing give artificial respiration. If breathing is difficult, give oxygen. If swallowed: Wash out mouth with water provided person is conscious. Wash contaminated .clothing before reuse. Discard contaminated shoes. IN ALL CASES CALL A PHYSICIAN OR POISON CONTROL CENTER (SEE PHONE NUMBER IN SECTION I). | Keep away from sources of ignition. NO SMOKING. |

# Reactivity Data

| Stability: STABLE | Conditions to avoid: NONE KNOWN |
| --- | --- |
| Incompatibility: NONE UNKNOWN | Materials to avoid: AROMATIC HYDROCARBONS |
| Hazard Polymerization: WILL NOT OCCUR | Hazardous Decomposition Products: NONE KNOWN |

# Environmental Protection Procedures

| Spill Response Procedures: Wear appropriate personal protective equipment, including but not limited to protective eyewear, clothing and gloves. Clean up with soap and water. | Waste Disposal Method: Normal Disposal Container Liquids: Wash down drain and flush with excess water.. |
| --- | --- |

# Handling and Special Protection Information

| Eye Protection: Wear safety glasses or full face shield | Skin Protection: Wear protective clothing and gloves. Remove all clothing if contaminated. |
| --- | --- |
| Respiratory: NONE REQUIRED | Ventilation: NORMAL |
| Other Precautions: NOT NEEDED |  |

# Special Precautions

Hygienic Practices in Handling and Storage: Carefully read PRECAUTIONS in the package insert instructions.

Precautions For Repair and Maintenance of Contaminated Material: NOT APPLICABLE

Other Precautions: For In Vitro Diagnostic Use Only. Do not use internally.

3 of 3